| S.No | Reference | Paper | Year | Methodology | Dataset | Performance Metrics | Key Findings | Research Gap |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | [1] | OpenVLA: An Open-Source Vision-Language-Action Model | 2024 | Open-source Vision-Language-Action (VLA) model for robotic manipulation | Open X-Embodiment | 70.6 ± 3.2% mean success rate across 29 tasks; 16.5% improvement over RT-2-X | Achieves state-of-the-art manipulation performance with efficient fine-tuning and open-source deployment | Requires high computational resources and lacks manufacturing-specific optimization |
| 2 | [2] | Vision-Language-Action Models for Robotics: A Review Towards Real-World Applications | 2025 | Comprehensive survey of VLA architectures, datasets, benchmarks and deployment strategies | Survey Literature | Reviews 100+ VLA studies and compares evaluation protocols | Summarizes recent advances, benchmarks and deployment challenges | No experimental implementation or quantitative evaluation |
| 3 | [3] | QUAR-VLA: Vision-Language-Action Model for Quadruped Robots | 2024 | Vision-Language-Action framework for autonomous navigation and manipulation on quadruped robots | QUARD (259K simulated + 3K real-world episodes) | Trained on 262K demonstrations with strong sim-to-real generalization | Enables language-guided locomotion, manipulation and reasoning | Focused on quadruped robots instead of collaborative industrial manipulators |
| 4 | [4] | Towards Open-World Grasping with Large Vision-Language Models | 2024 | VLM integrated with segmentation, grasp synthesis and contact reasoning | OCID-VLG, Grasp-MultiObject, Real Robot Evaluation | Outperforms previous supervised and zero-shot LLM baselines in simulation and real hardware | Improves zero-shot grasp planning for unseen objects | Limited to grasping rather than complete manufacturing workflows |
| 5 | [5] | Vision-Language Model-Based Human-Robot Collaboration for Smart Manufacturing: A State-of-the-Art Survey | 2025 | Systematic survey of VLMs in collaborative manufacturing | 109 selected research papers | Reviews 109 publications covering assembly, inspection and HRC | Provides taxonomy, industrial challenges and future research directions | No implementation or quantitative benchmarking |
| 6 | [6] | Vision-Language Models for Human-Robot Collaboration | 2025 | Review of multimodal perception, language understanding and collaborative robotics | Human-Robot Collaboration Literature | Comparative analysis of recent VLM-based HRC frameworks | Summarizes multimodal reasoning and collaborative task execution | Limited discussion on manufacturing deployment and industrial validation |